r7

router bgp 65046

router-id 10.46.255.255

bgp log-neighbor-changes

neighbor 10.46.255.254 remote-as 65046

neighbor 10.46.255.254 next-hop-self

neighbor 10.46.255.254 update-source Loopback0

neighbor 10.46.255.254 password lab123!

router-id 10.46.255.254

r7 ja r8

router bgp 65046

route-reflector preserve-attributes

neighbor 10.46.255.254 remote-as 65046

neighbor 10.46.255.254 next-hop-self

neighbor 10.46.255.254 update-source Loopback0

neighbor 10.46.255.254 password lab123!

neighbor RRCLIENT peer group

neighbor RRCLIENT remote-as 65046

neighbor RRCLIENT next-hop-self

neighbor RRCLIENT update-source Loopback0

neighbor RRCLIENT password lab123!

neighbor 10.46.255.1 peer group RRCLIENT

neighbor 10.46.255.2 peer group RRCLIENT

neighbor 10.46.255.3 peer group RRCLIENT

neighbor 10.46.255.4 peer group RRCLIENT

neighbor 10.46.255.5 peer group RRCLIENT

neighbor 10.46.255.6 peer group RRCLIENT

r1-r6

router bgp 65046

router-id 10.46.255.1

bgp log-neighbor-changes

neighbor RR update-source loopback0

neighbor RR next-hop-self

neighbor RR password lab123!

neighbor 10.46.255.254 peer group RR

neighbor 10.46.255.255 peer group RR

redistribute connected

R7 ja r8

router bgp 65046

neighbor RRCLIENT route-reflector-client

cluster id

r7

router pgb	65046

bgp cluster-id 10.46.255.0