2220

sh vlan

sh route

sh interfaces vlan2

sh mac address-table vlan 2

Kaikille als1-4

spanning-tree mode rpvst

spanning-tree

spanning-tree vlan 1,2

Kaikkiin dls1-4

spanning-tree mode rapid-pvst

Dls1 ja 3

spanning-tree vlan-id 1-4094 root primary

dls2 ja 4

spanning-tree vlan-id 1-4094 root secondary

Step 2

NN46-DLS1(config)#int vlan 2

NN46-DLS1(config-if-Vl2)#vrrp 2 priority-level 120

NN46-DLS2(config)#int vlan 2

NN46-DLS2(config-if-Vl2)#vrrp 2 priority-level 80

ALS1

vlan 5

name OFFICE

exit

spanning-tree vlan 5

DLS jakelu muille DLS1 konffi

interface Vlan5

ip address 10.46.105.251/24

isis enable LAN

isis passive

vrrp 5 priority-level 120

vrrp 5 ipv4 10.46.105.1

!

Secondary DLS

interface Vlan2

ip address 10.46.102.252/24

isis enable LAN

isis passive

vrrp 2 priority-level 80

vrrp 2 ipv4 10.46.102.1

Muuta 2 ->5

interface Vlan5

ip address 10.46.105.252/24

isis enable LAN

isis passive

vrrp 5 priority-level 80

vrrp 5 ipv4 10.46.105.1

DLS3

interface Vlan5

ip address 10.46.205.252/24

isis enable LAN

isis passive

vrrp 5 priority-level 120

vrrp 5 ipv4 10.46.205.1

!

DLS4

interface Vlan5

ip address 10.46.205.252/24

isis enable LAN

isis passive

vrrp 5 priority-level 80

vrrp 5 ipv4 10.46.205.1

!

vlan 5

name OFFICE

spanning-tree vlan 5

Step2 check

ALS1-3

int 1/1/3

vlan access 5

no shutdown

ALS4

vlan trunk allowed all

vlan trunk native 2

DC1

ping 10.46.202.1

Kaikille als1-4

ALS1

spanning-tree mode rpvst

spanning-tree

spanning-tree vlan 1,2

vlan 5

name OFFICE

exit

spanning-tree vlan 5

int 1/1/3

vlan access 5

no shutdown

ALS2

spanning-tree mode rpvst

spanning-tree

spanning-tree vlan 1,2

vlan 5

name OFFICE

exit

spanning-tree vlan 5

int 1/1/3

vlan access 5

no shutdown

ALS3

spanning-tree mode rpvst

spanning-tree

spanning-tree vlan 1,2

vlan 5

name OFFICE

exit

spanning-tree vlan 5

int 1/1/3

vlan access 5

no shutdown

ALS4

spanning-tree mode rpvst

spanning-tree

spanning-tree vlan 1,2

vlan 5

name OFFICE

exit

spanning-tree vlan 5

int 1/1/3

vlan trunk allowed all

vlan trunk native 2

DLS1

spanning-tree mode rapid-pvst

spanning-tree vlan-id 1-4094 root primary

int vlan 2

vrrp 2 priority-level 120

vlan 5

name OFFICE

interface Vlan5

ip address 10.46.105.251/24

isis enable LAN

isis passive

vrrp 5 priority-level 120

vrrp 5 ipv4 10.46.105.1

DLS2

spanning-tree mode rapid-pvst

spanning-tree vlan-id 1-4094 root secondary

int vlan 2

vrrp 2 priority-level 80

vlan 5

name OFFICE

interface Vlan5

ip address 10.46.105.251/24

isis enable LAN

isis passive

vrrp 5 priority-level 80

vrrp 5 ipv4 10.46.105.1

DLS3

spanning-tree mode rapid-pvst

spanning-tree vlan-id 1-4094 root primary

int vlan 2

vrrp 2 priority-level 120

vlan 5

name OFFICE

interface Vlan5

ip address 10.46.205.252/24

isis enable LAN

isis passive

vrrp 5 priority-level 120

vrrp 5 ipv4 10.46.205.1

DLS4

spanning-tree mode rapid-pvst

spanning-tree vlan-id 1-4094 root secondary

int vlan 2

vrrp 2 priority-level 80

vlan 5

name OFFICE

interface Vlan5

ip address 10.46.205.252/24

isis enable LAN

isis passive

vrrp 5 priority-level 80

vrrp 5 ipv4 10.46.205.1

PCA

ping

10.46.105.50