Spine

router isis DC

net 49.0000.0000.0001.0003.00

is-type level-2

log-adjacency-changes

!

address-family ipv4 unicast

metric 10000

!

!

interface Loopback0

ip address 10.46.1.3/32

isis enable DC

isis passive

!

interface ethernet 1-4

ip address unnumbered Lo0

mtu 1600

isis enable DC

isis network point-to-point

!

Leaf

router isis DC

net 49.0000.0000.0002.0001.00

is-type level-2

log-adjacency-changes

!

address-family ipv4 unicast

metric 10000

!

!

interface Loopback0

ip address 10.46.2.1/32

isis enable DC

isis passive

!

interface ethernet1-3

ip address unnumbered Lo0

mtu 1600

isis enable DC

isis network point-to-point

!

!

configure vxlan

L1, L4, L3, L2

interface vxlan1

vxlan source-interface loopback0

vxlan vlan 10 vni 1010

vxlan flood vtep 10.46.2.1 10.46.2.2 10.46.2.3 10.46.2.4