3220

SPINE:

router bgp 65460

router-id 10.46.1.3

bgp listen range 10.46.0.0/16 peer-group LEAF remote-as 65461

neighbor LEAF peer group

network 10.46.1.3/32

LEAF 1:

router bgp 65461

router-id 10.46.1.1

neighbor SPINE peer group

neighbor SPINE remote-as 65460

neighbor 10.46.11.1 peer group SPINE

neighbor 10.46.21.1 peer group SPINE

neighbor 10.46.31.1 peer group SPINE

network 10.46.1.1/32

lEAf 2:

router bgp 65461

router-id 10.46.2.4

maximum-paths 8 ecmp 8

neighbor SPINE peer group

neighbor SPINE remote-as 65460

neighbor SPINE allowas-in 1

neighbor 10.46.14.1 peer group SPINE

neighbor 10.46.24.1 peer group SPINE

neighbor 10.46.34.1 peer group SPINE

network 10.46.2.4/32

neighbour SPINE allowas-in

Step2

router bgp 65460

neighbor LEAF send-community extended

!

address-family evpn

neighbor LEAF activate

!

router bgp 65461

neighbor SPINE send-community extended

!

address-family evpn

neighbor SPINE activate

!

interface Vxlan1

vxlan source-interface Loopback0

vxlan vlan 10 vni 1010

!

router bgp 65461

1:10.10.10.0/24

2:10.10.10.0/24

1:00:02:ab:ba:cd:00

2:00:02:ab:ba:cd:00

interface Vxlan1

vxlan source-interface Loopback0

vxlan vlan 10 vni 1010

!

router bgp 65461

vlan 10

rd 10.46.2.4:1010

route-target both 65460:1010

redistribute learned

!

!