Spine template

conf t

route-map SRCIP permit 10

set src 10.0.0.3

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 65046

bgp router-id 10.0.0.0

no bgp ebgp-requires-policy

neighbor LEAF peer-group

neighbor LEAF remote-as external

neighbor eth1 interface peer-group LEAF

neighbor eth2 interface peer-group LEAF

neighbor eth3 interface peer-group LEAF

neighbor eth4 interface peer-group LEAF

neighbor eth5 interface peer-group LEAF

neighbor eth6 interface peer-group LEAF

neighbor eth7 interface peer-group LEAF

neighbor eth8 interface peer-group LEAF

!

address-family ipv4 unicast

redistribute connected route-map DCLB

end

wr mem

Leaf template

route-map SRCIP permit 10

set src [LOIP]

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 651[NN]

bgp router-id 10.1.0.0

no bgp ebgp-requires-policy

neighbor HOST peer-group

neighbor HOST remote-as external

neighbor SPINE peer-group

neighbor SPINE remote-as external

neighbor eth1 interface peer-group SPINE

neighbor eth2 interface peer-group SPINE

neighbor eth3 interface peer-group SPINE

neighbor eth4 interface peer-group HOST

neighbor eth5 interface peer-group HOST

!

address-family ipv4 unicast

neighbor SPINE allowas-in 1

redistribute connected route-map DCLB

Host template

route-map SRCIP permit 10

set src [LOIP]

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 652NN

bgp router-id 10.2.0.0

no bgp ebgp-requires-policy

neighbor LEAF peer-group

neighbor LEAF remote-as external

neighbor eth1 interface peer-group LEAF

neighbor eth1 capability extended-nexthop

neighbor eth2 interface peer-group LEAF

neighbor eth2 capability extended-nexthop

!

address-family ipv4 unicast

neighbor LEAF allowas-in 1

redistribute connected route-map DCLB

Leaf template

conf t

route-map SRCIP permit 10

set src 10.1.2.2

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 65146

bgp router-id 10.1.0.0

no bgp ebgp-requires-policy

neighbor HOST peer-group

neighbor HOST remote-as external

neighbor SPINE peer-group

neighbor SPINE remote-as external

neighbor eth1 interface peer-group SPINE

neighbor eth2 interface peer-group SPINE

neighbor eth3 interface peer-group SPINE

neighbor eth4 interface peer-group HOST

neighbor eth5 interface peer-group HOST

!

address-family ipv4 unicast

neighbor SPINE allowas-in 1

redistribute connected route-map DCLB

end

wr mem

sh bgp summary

conf t

route-map SRCIP permit 10

set src 10.1.2.3

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 65146

bgp router-id 10.1.0.0

no bgp ebgp-requires-policy

neighbor HOST peer-group

neighbor HOST remote-as external

neighbor SPINE peer-group

neighbor SPINE remote-as external

neighbor eth1 interface peer-group SPINE

neighbor eth2 interface peer-group SPINE

neighbor eth3 interface peer-group SPINE

neighbor eth4 interface peer-group HOST

neighbor eth5 interface peer-group HOST

!

address-family ipv4 unicast

neighbor SPINE allowas-in 1

redistribute connected route-map DCLB

end

wr mem

MOlemmat l4A ja B

conf t

route-map SRCIP permit 10

set src 10.1.2.4

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 65146

bgp router-id 10.1.0.0

no bgp ebgp-requires-policy

neighbor HOST peer-group

neighbor HOST remote-as external

neighbor SPINE peer-group

neighbor SPINE remote-as external

neighbor eth1 interface peer-group SPINE

neighbor eth2 interface peer-group SPINE

neighbor eth3 interface peer-group SPINE

!

address-family ipv4 unicast

neighbor SPINE allowas-in 1

redistribute connected route-map DCLB

end

wr mem

PVE1

vtysh

host template

set src 10.2.2.1 muuta   10.2.1.1, 10.2.1.2

1.

conf t

route-map SRCIP permit 10

set src 10.2.2.1

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 65246

bgp router-id 10.2.0.0

no bgp ebgp-requires-policy

neighbor LEAF peer-group

neighbor LEAF remote-as external

neighbor eth1 interface peer-group LEAF

neighbor eth1 capability extended-nexthop

neighbor eth2 interface peer-group LEAF

neighbor eth2 capability extended-nexthop

!

address-family ipv4 unicast

neighbor LEAF allowas-in 1

redistribute connected route-map DCLB

end

wr mem

PVE1

vtysh

host template

1.

conf t

route-map SRCIP permit 10

set src 10.2.2.3

!

ip protocol bgp route-map SRCIP

!

ip prefix-list DCLB permit 10.0.0.0/8 ge 32

!

route-map DCLB permit 10

match ip address prefix-list DCLB

!

router bgp 65246

bgp router-id 10.2.0.0

no bgp ebgp-requires-policy

neighbor LEAF peer-group

neighbor LEAF remote-as external

neighbor eth1 interface peer-group LEAF

neighbor eth1 capability extended-nexthop

neighbor eth2 interface peer-group LEAF

neighbor eth2 capability extended-nexthop

!

address-family ipv4 unicast

neighbor LEAF allowas-in 1

redistribute connected route-map DCLB

end

wr mem

L4a

int eth4

ip address 10.14.11.1/24

int eth5

ip address  10.14.12.1/24

l4b

eth4

10.14.21.1/24

eth5

10.14.22.1/24