Conf t

Interface loopback0

Description loopback

Ip ospf area 0.0.0.0

!

Router ospf 1

Router-id 10.29.0.4

Auto-cost reference-bandwidth 100000

Passive-interface default

No passive-interface Ethernet 2-3

!

Interface ethernet 2-3

Ip ospf network point-to-point

Ip address unnumbered Loopback0

Ip ospf area 0.0.0.0

Ip ospf authentication message-digest

Ip ospf message digest-key 1 sha512 lab123!

!

!Interface Ethernet 4

!Ip ospf area 0.0.0.0

!

R1

Router bgp

Router bgp 1029

Bqp loq-neighbour-changes

Neighbor IBGP peer group

Neighbor IBGP remote-as 1029

Neighbor IBGP update-source Loopback0

Neighbor 10.29.0.2 peer group IBGP

Neighbor 10.29.0.3 peer group IBGP

Neighbor 10.29.0.4 peer group IBGP

Step2

Interface eth 1

R1

Ip address 100.64.1.46/24

Pinggaa 100.64.1.46

Router bgp 1029

Neighbor 100.64.1.2 remote-as 1429

Neighbor 100.64.1.2 password lab123!

Neighbor 100.64.1.2 ttl maximum hops 1

Mainosta

Router bgp 1029

Network 10.29.1.0/24

Router bgp 1029

neyw

R2

Ip address 100.64.2.46/24

Pinggaa 100.64.2.46

Router bgp 1029

Neighbor 100.64.1.2 remote-as 1429

Neighbor 100.64.1.2 password lab123!

Neighbor 100.64.1.2 ttl maximum hops 1

Mainosta

Router bgp 1029

Network 10.29.2.0/24

R4

Neighbor 10.29.0.1 peer group IBGP

Neighbor 10.29.0.2 peer group IBGP

Neighbor 10.29.0.3 peer group IBGP

Viimeinen silaus

Ip route 10.29.0.0/21 Null0

Router bgp 1029

Network 10.29.0.0/21